# IT Infrastructure

Network Team: -

- Youssef Mohamed Hussein     2023/03820

- Malak Mostafa Zaki                  2023/04840

- Dana Sameh Mohamed           2023/08899

- Jomana Yasser Mohamed Aly 2023/04423

Servers Team: -

- Aida Bahaa Abdelmesieh         2023/06836

- Saja Hany                                    2023/05663

- Malak Akram                              2023/00997

- Mennatallah Osama                 2023/06241

Penetration Test

- Ayah Ahmed Yousef                  2023/05340

- Shahd Mahmoud Hassanein   2023/02021

Ethical hacking

- Youssef Ahmed Salah               2023/07736

- Ahmed Mohamed Ahmed       2023/04661

- Adam Ehab Mamdouh             2023/01176

- Abanoub Atef                             2023/10131

- Ahmed Mohamed Sobhi          2023/01261

- Salma Yasser                               2023/00364

Contents

Network Planning and Addressing Design	3

Core Switch (Cisco 3560 catalyst)	4

Step 1: Enter Configuration Mode & Set Hostname: -	4

Step 2: Configure DHCP Excluded Addresses: -	4

Step 3: Configure DHCP for Wired Users (VLAN 10): -	4

Step 4: Configure DHCP for Wireless Users (VLAN 20): -	4

Step 5: Configure VLAN Interfaces (SVI): -	5

Step 6: Create VLANs: -	5

Step 7: Configure Trunk Link to Access Switch: -	5

Step 8: Configure EtherChannel: -	6

Step 9: Configure Port-Channel: -	6

Step 10: Configure ACL for Server Security: -	6

Step 11: Apply ACL to VLAN 30: -	6

Step 12: Configure Spanning Tree: -	6

Step 13: Verification Commands: -	7

Access Switch Configuration (Juniper EX3200)	10

Step 1: Enter Configuration Mode: -	10

Step 2: Create VLANs: -	10

Step 3: Configure Wired Client Ports (VLAN 10): -	10

Step 4: Configure Access Point (Wireless VLAN 20): -	10

Step 5: Configure Trunk Link to Core Switch: -	10

Step 6: Configure Spanning Tree: -	10

Step 7: Configure DHCP Relay: -	11

Step 8: Apply Configuration: -	11

Server Switch Configuration (Cisco Catalyst 3560)	15

Step 1: Hostname Configuration: -	15

Step 2: VLAN Creation: -	15

Step 3: Port Assignment: -	15

Step 4: Trunk Configuration: -	16

Step 5: EtherChannel Configuration: -	16

Step 6: Spanning Tree Protocol (STP): -	16

Step 7: Save Configuration: -	16

Step 8: Verification: -	16

# Network Planning and Addressing Design

Network address: - 172.16.0.0/24

IPV4 32 bits  32-24 = 8 host for network

- 11111111. 11111111. 11111111. 00000000 = 255.255.255.0

VLANs

| Wired users | Wired users | Servers |
| --- | --- | --- |
| VLAN 10 | VLAN 20 | VLAN 30 |

According to this, we have 3 VLANs so we need 3 subnets 
Subnet = 2^n      2^2 =4 subnets so, we need 2 bits
Number of usable hosts per subnet = 2^h-2
h  number of hosts = 2^6 - 2 = 64 – 2 = 62 host

Network address: - 172.16.0.0/26

- 255.255.255.11000000  255.255.255.192

|  | Network | First usable | Last Usable | Broadcast | Gateway |
| --- | --- | --- | --- | --- | --- |
| VLAN 10 | 172.16.0.0/26 | 172.16.0.1 | 172.16.0.62 | 172.16.0.63 | 172.16.0.1 |
| VLAN 20 | 172.16.0.64/26 | 172.16.0.65 | 172.16.0.126 | 172.16.0.127 | 172.16.0.65 |
| VLAN 30 | 172.16.0.128/26 | 172.16.0.129 | 172.16.0.190 | 172.16.0.191 | 172.16.0.129 |

# Core Switch (Cisco 3560 catalyst)

## Step 1: Enter Configuration Mode & Set Hostname: -

- enable

- configure terminal

- hostname CORE-SW

We access privileged mode and configure the switch name for identification in the network.

## Step 2: Configure DHCP Excluded Addresses: -

- ip dhcp excluded-address 172.16.0.1 172.16.0.9

- ip dhcp excluded-address 172.16.0.65 172.16.0.69

- ip dhcp excluded-address 172.16.0.129 172.16.0.139

These IPs are reserved for gateways and servers and will not be assigned dynamically to clients.

## Step 3: Configure DHCP for Wired Users (VLAN 10): -

- ip dhcp pool wired_users

- network 172.16.0.0 255.255.255.192

- default-router 172.16.0.1

- dns-server 8.8.8.8

- exit

This creates a DHCP pool for wired users and assigns IP addresses automatically.

## Step 4: Configure DHCP for Wireless Users (VLAN 20): -

- ip dhcp pool wireless_users

- network 172.16.0.64 255.255.255.192

- default-router 172.16.0.65

- dns-server 8.8.8.8

- exit

This pool provides IP addresses for wireless clients connected through the access point.

## Step 5: Configure VLAN Interfaces (SVI): -

- interface vlan 10

- ip address 172.16.0.1 255.255.255.192

- no shutdown

- exit

- interface vlan 20

- ip address 172.16.0.65 255.255.255.192

- no shutdown

- exit

- interface vlan 30

- ip address 172.16.0.129 255.255.255.192

- no shutdown

- exit

These interfaces act as gateways for each VLAN and allow inter-VLAN routing.

## Step 6: Create VLANs: -

- vlan 10

- name wired_users

- exit

- vlan 20

- name wireless_users

- exit

- vlan 30

- name servers

- exit

VLANs are created to separate network traffic into logical groups.

## Step 7: Configure Trunk Link to Access Switch: -

- interface gigabitEthernet0/1

- description LINK_TO_ACCESS_SWITCH

- switchport trunk encapsulation dot1q

- switchport mode trunk

- switchport trunk allowed vlan 10,20,30

- exit

This port carries multiple VLANs between switches.

## Step 8: Configure EtherChannel: -

- interface range gi0/1 - 2

- channel-group 1 mode active

- no shutdown

- exit

Combines multiple physical links into one logical link for redundancy and higher bandwidth.

## Step 9: Configure Port-Channel: -

- interface port-channel 1

- switchport trunk encapsulation dot1q

- switchport mode trunk

- switchport trunk allowed vlan 10,20,30

- exit

This is the logical interface created by EtherChannel.

## Step 10: Configure ACL for Server Security: -

- ip access-list extended SERVER_SECURITY

- permit tcp 172.16.0.0 0.0.0.127 172.16.0.128 0.0.0.63 eq 80

- permit tcp 172.16.0.0 0.0.0.127 172.16.0.128 0.0.0.63 eq 443

- deny ip 172.16.0.0 0.0.0.127 host 172.16.0.150

- permit ip any any

- exit

Allows users to access web servers but blocks direct access to the database server.

## Step 11: Apply ACL to VLAN 30: -

- interface vlan 30

- ip access-group SERVER_SECURITY out

- exit

Applies the security rules to traffic leaving the server VLAN.

## Step 12: Configure Spanning Tree: -

- spanning-tree mode rapid-pvst

- spanning-tree vlan 10,20,30 priority 4096

Prevents network loops and makes this switch the root bridge.

## Step 13: Verification Commands: -

- show ip interface brief

- show vlan brief

- show interfaces trunk

- show etherchannel summary

- show access-lists

- show spanning-tree vlan 10

Used to verify that all configurations are working correctly.

# Access Switch Configuration (Juniper EX3200)

## Step 1: Enter Configuration Mode: -

- cli

- configure

This command allows us to enter configuration mode to apply settings on the switch.

## Step 2: Create VLANs: -

- set vlans WIRED vlan-id 10

- set vlans WIRELESS vlan-id 20

- set vlans SERVERS vlan-id 30

We create VLANs to logically separate wired users, wireless users, and servers.

## Step 3: Configure Wired Client Ports (VLAN 10): -

- set interfaces ge-0/0/1 unit 0 family ethernet-switching interface-mode access

- set interfaces ge-0/0/1 unit 0 family ethernet-switching vlan members WIRED

- set interfaces ge-0/0/2 unit 0 family ethernet-switching interface-mode access

- set interfaces ge-0/0/2 unit 0 family ethernet-switching vlan members WIRED

- repeated steps till ge-0/0/8

These ports are configured as access ports and assigned to VLAN 10 for wired users.

## Step 4: Configure Access Point (Wireless VLAN 20): -

- set interfaces ge-0/0/9 unit 0 family ethernet-switching interface-mode access

- set interfaces ge-0/0/9 unit 0 family ethernet-switching vlan members WIRELESS

This port connects the access point and assigns it to the wireless VLAN.

## Step 5: Configure Trunk Link to Core Switch: -

- set interfaces ge-0/0/24 unit 0 family ethernet-switching interface-mode trunk

- set interfaces ge-0/0/24 unit 0 family ethernet-switching vlan members [ WIRED WIRELESS SERVERS ]

This trunk port carries all VLAN traffic between the access switch and the core switch.

## Step 6: Configure Spanning Tree: -

- set protocols rstp interface ge-0/0/1 edge

- set protocols rstp interface ge-0/0/2 edge

- repeated steps till ge-0/0/9

Edge ports are configured for faster connection and to prevent STP delays on end devices.

## Step 7: Configure DHCP Relay: -

- set forwarding-options helpers bootp server 172.16.0.1

This forwards DHCP requests from clients to the Core switch where DHCP is configured.

## Step 8: Apply Configuration: -

- commit

This command saves and applies all configuration changes.

# Server Switch Configuration (Cisco Catalyst 3560)

## Step 1: Hostname Configuration: -

- enable

- configure terminal

- hostname SERVER-SW

Setting a unique hostname helps identify the device easily during troubleshooting.

## Step 2: VLAN Creation: -

- vlan 10

- name WIRED_USERS

- exit

- vlan 20

- name WIRELESS_USERS

- exit

- vlan 30

- name SERVERS

- exit

VLANs are created to logically separate wired users, wireless users, and servers.

## Step 3: Port Assignment: -

- interface range gi0/1 - 4

- switchport mode access

- switchport access vlan 30

- no shutdown

- interface range gi0/5 - 6

- switchport mode access

- switchport access vlan 10

- no shutdown

- interface range gi0/7 - 8

- switchport mode access

- switchport access vlan 20

- no shutdown

Ports are assigned based on device type:

- VLAN 30 → Servers

- VLAN 10 → Wired users

- VLAN 20 → Wireless users

## Step 4: Trunk Configuration: -

- interface range gi0/5 – 6

- description LINK_TO_CORE

- switchport mode trunk

- switchport trunk allowed vlan 10,20,30

These ports now act as uplinks carrying all VLANs (10, 20, 30) between switches.

## Step 5: EtherChannel Configuration: -

- interface range gi0/5 – 6

- channel-group 1 mode active

- interface port-channel 1

- switchport mode trunk

- switchport trunk allowed vlan 10,20,30

Gi0/5 and Gi0/6 are bundled into one logical link (Port-Channel1) for:

- Higher bandwidth

- Redundancy (if one link fails, the other stays active)

## Step 6: Spanning Tree Protocol (STP): -

- spanning-tree mode rapid-pvst

RSTP prevents loops and ensures fast convergence in the network.

## Step 7: Save Configuration: -

- write memory

Saves the configuration so it remains after reboot.

## Step 8: Verification: -

- show vlan brief

- show etherchannel summary

- show spanning-tree